# ANSHUMAN — Comprehensive Design & Development Guide

AI-Based LPI Waveform Generation • SDR-Ready • End-to-End Blueprint

Table of Contents (Right-click → Update Field in Word)

1. Mission objectives, scope & deliverables

2. Requirements (functional & non-functional)

3. Threat model for LPI (what the adversary can do)

4. System architecture (Diagram B1)

5. Technology choices & “why not X” rationale

6. TX & RX signal pipelines (Diagram B3)

7. AI/ML design (G, Dec, D, Classifier), losses, curriculum (Diagram B2)

8. Security & keying plan

9. SDR integration (file formats, flowgraphs, calibration)

10. Test & validation plan (BER, AUROC, PSD gap; OTA steps)

11. Risk register & mitigations

12. Roadmap (Phase‑0 → Phase‑3)

13. BOM, software stack, field SOPs

14. Troubleshooting & FAQ

15. Glossary

16. Code mapping and parameter appendix

# 1) Mission objectives, scope & deliverables

Transmit hidden information within noise-like I/Q waveforms (Low Probability of Intercept, LPI).

Maintain reliable recovery at friendly receiver with low BER using FEC.

Defend against classical detectors (energy, cyclostationary) and AI detectors (CNN on spectrograms).

Integrate with SDRs (BladeRF/USRP) for lab and OTA trials.

Deliver modular codebase, documentation, KPIs, and flowgraphs for field use.

# 2) Requirements (functional & non-functional)

Functional Requirements:

Embed RS-coded bits in noise-like I/Q frames.

Receiver reconstructs bits with BER ≤ 1% at SNR ≥ 15 dB (tunable).

Export float32 interleaved I/Q .bin files for SDR playback/transmit.

No obvious preambles or signatures that enable easy detection.

Non-Functional Requirements:

Stealth: detector AUROC ≤ 0.6 vs baseline CNN at target SNRs.

Robustness: tolerate AWGN + CFO ±200 Hz, ±50 ppm clock offsets (lab).

Reproducibility: deterministic seeds; configuration files for all hyperparameters.

Usability: simple CLI, SOP checklists, clear logs and metrics.

# 3) Threat model for LPI (what the adversary can do)

Wideband energy detection, spectral kurtosis, and cyclostationary analysis.

Supervised CNN-based spectrogram detectors trained on common modulations.

Offline analysis of captured I/Q; adaptive thresholds and multi-sensor fusion.

Active measures: jamming, probing, replay; channel state estimation attempts.

# 4) System architecture (Diagram B1)

Preprocess: XOR mask (demo) + Reed–Solomon (RS) FEC.

Latent z: randomness to avoid repetitive structures; essential for stealth.

Generator: conditional MLP maps (z, bits) → tanh-bounded I/Q.

Channel: AWGN in baseline; extend with fading, CFO, timing offsets.

Decoder: neural demapper recovers RS bitstream; RS decode + unmask → plaintext.

Scope/Implementation flow with AI/ML emphasis (S1–S9).

# 5) Technology choices & “why not X” rationale

Chosen: PyTorch for G/Dec/D/C; numpy for bit ops; reedsolo for RS; matplotlib for diagrams; GNU Radio for SDR.

Chosen: .bin float32 I/Q as interchange format (simple, fast, device-agnostic).

Not now: OFDM and standard modems—introduce cyclostationary signatures; easier to detect.

Not now: LDPC—higher coding gain but greater complexity; RS suffices for baseline.

Not now: Full AES-GCM integration—kept out to reduce scope creep; earmarked for Phase‑2.

Not now: CNN adversaries—start with MLPs for rapid iteration; add CNNs in Phase‑2.

# 6) TX & RX signal pipelines (Diagram B3)

TX: Plaintext → bitpack → XOR mask → RS encode → z sample → Generator → normalize/interleave → .bin export → GNU Radio TX.

RX: SDR capture → DC/AGC/filter → frame to Decoder → bit logits → threshold → RS decode → XOR unmask → plaintext.

# 7) AI/ML design (G, Dec, D, Classifier), losses, curriculum (Diagram B2)

Core modules:

Generator (MLP): conditional mapping (z, RS bits) → I/Q (tanh).

Decoder (MLP): I/Q → RS bits (sigmoid); replaces classical demodulator.

Discriminator (MLP): real-noise vs generated; adversarial critic.

Signal Classifier (MLP): detector adversary; encourages stealth.

Training objectives:

Reconstruction (BCE) on bits at Decoder output.

Adversarial (BCE): G fools D; detection loss (CE): G appears as noise to C.

Curriculum: start at high SNR; gradually lower SNR; alternate updates.

Next-step improvements: WGAN-GP/hinge loss, gradient penalty, feature matching, spectral (PSD/KL) losses.

# 8) Security & keying plan (present and deployment-grade)

Present (lab): XOR mask for demo; single shared key per session.

Deployment: AES-GCM or ChaCha20-Poly1305; per-frame nonce/IV; MAC for integrity.

Key management: KDF for session keys; rekey schedule; zeroization; secure storage.

Optional: cryptographic PRNG to drive frequency hopping/spreading and timing jitter.

# 9) SDR integration (file formats, flowgraphs, calibration)

File format: float32 interleaved I/Q (|I|,|Q| ≤ 1).

Typical lab rates: 1–4 MS/s; calibrate TX gain to meet spectral mask.

GNU Radio TX: File Source → (Resampler/Throttle) → LPF → Sink (BladeRF/USRP).

GNU Radio RX: Source → DC Block → AGC → LPF → File Sink → Decoder script.

Calibration: remove DC; compensate IQ imbalance; capture known test sequences for lab-only checks.

# 10) Test & validation plan (BER, AUROC, PSD gap; OTA steps)

Unit tests: bit utilities (pack/unpack), RS encode/decode, XOR mask/unmask.

Model checks: forward/backward sanity, overfit tiny dataset to verify plumbing.

Channel sweep: BER & AUROC vs SNR (30→0 dB), CFO, timing offsets; log PSD gap/KL to noise.

Detectors: radiometer, cyclostationary, baseline CNN—plot Pd vs Pfa and AUROC.

OTA ladder: coax loopback → shielded near-field → controlled open area; record EVM/BER.

# 11) Risk register & mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| GAN instability/mode collapse | Training fails to converge | WGAN-GP/hinge, grad penalty, schedulers |
| Overfitting to AWGN | Poor OTA performance | Richer impairments; augmentation; curriculum |
| Detector generalization | Detected in field | Multi-detector training; red-team tests |
| Key management errors | Compromised confidentiality | AEAD, KDF, rekey, SOPs |
| SDR impairments | Higher BER/PSD anomalies | Model impairments; calibrate and compensate |

# 12) Roadmap (Phase‑0 → Phase‑3)

Phase‑0 (Now): AWGN baseline, RS FEC, XOR masking, MLPs; .bin export; basic BER logging.

Phase‑1: Add WGAN‑GP/hinge, PSD/KL losses; expand channel (Rayleigh, CFO, timing jitter).

Phase‑2: CNN detectors; AES‑GCM; implicit sync; ablations and full AUROC benchmarking.

Phase‑3: GNU Radio OTA demos; field SOPs; HIL testing; hardening and report pack.

# 13) BOM, software stack, field SOPs

Bill of Materials (example lab):

| Item | Qty | Notes |
| --- | --- | --- |
| BladeRF or USRP B200/B205 | 2 | TX/RX nodes |
| RF cables/attenuators | Assorted | Loopback, isolation |
| Laptop/NUC (USB 3.0) | 2 | GNU Radio + Python |
| (Optional) Spectrum Analyzer | 1 | PSD verification |

Software stack:

Python 3.x; PyTorch; numpy; reedsolo; matplotlib.

GNU Radio 3.9+ with BladeRF/USRP drivers.

Config file (YAML/JSON): RS_NSYM, SEQ_LENGTH, LATENT_DIM, SNR, frame size.

Field SOPs (abridged):

Load keys/config; verify versions.

Generate frames and .bin; stage in GNU Radio TX.

Set conservative TX gain; verify spectral mask.

Start RX capture; monitor BER; rotate session keys.

Archive logs/configs post-mission.

# 14) Troubleshooting & FAQ

High BER → increase RS_NSYM, reduce rate, train at lower SNRs.

Detector alarms → add PSD/KL loss, train against stronger detectors.

Clipping → check normalization and TX gain, add limiter.

CFO drift → implement coarse/fine frequency correction in RX.

Mismatched parameters → verify frame size, RS settings, and seeds on both ends.

# 15) Glossary

| Term | Meaning |
| --- | --- |
| LPI | Low Probability of Intercept |
| I/Q | In-phase and Quadrature baseband components |
| AWGN | Additive White Gaussian Noise |
| RS | Reed–Solomon error-correcting code |
| AEAD | Authenticated Encryption with Associated Data |
| AUROC | Area Under ROC (detector performance) |
| EVM | Error Vector Magnitude (signal quality) |

# 16) Code mapping and parameter appendix

Preprocessing: xor_mask, rs_encode, rs_decode.

Models: Generator, Decoder, Discriminator, SignalClassifier classes.

Channel: add_awgn (extendable to fading/CFO).

Training: loops optimizing recon+adv+det losses; logging of BER/accuracy.

Export: encode_and_export_chunk, message_to_chunks for .bin files.

Default parameters (tunable):

| Parameter | Default | Notes |
| --- | --- | --- |
| RS_NSYM | 16 | More parity → better correction, lower throughput |
| LATENT_DIM | 16 | Higher → more diversity; tune for stability |
| SEQ_LENGTH | ≥ 2×RS_TOTAL_BITS | Frames per I/Q sequence |
| SNR (train) | 20 dB | Use curriculum 25→10 dB |
| FRAME_CHARS | 4 | Adjust throughput/BER |